Software required for development with the Sumovore PIC Brain board Rev. 2.0

MPLAB IDE (http://ww1.microchip.com/downloads/en/DeviceDoc/mp740_full.zip)

Free Download

HI TECH PICC-Lite  (http://www.htsoft.com/products/PICClite_comparison.php)

Free Download with site registration

PIC Downloader (Solarbotics website resources)

Free Download

Optional:

Sumoline_2.mcp (Solarbotics website resources)

Free Download

First install the MPLAB IDE software to your computer. (restart)

Then install Hi Tech's PICC-Lite software which will integrate with the MPLAB suite.

Unzip the PIC Downloader and extract it to your hard drive.

Drag the "PIC BootPlus" shortcut to your desktop for easy access.

Now start the MPLAB IDE program (via start menu) and then you can open the

Sumoline_2.mcp project file and edit it or create your own project from scratch. Hit the Project/Make button (on top toolbar) (F10) to compile your C code to a hex file.

Connect your robot to a serial link with the cable you made.

Start the PIC BootPlus downloader by double clicking on the icon you put on your desktop.

Find the hex file you created by clicking the "Search" button.

Select the com port you are using.

Set the Baud rate to 38400bps.

Download the hex file to your robot by clicking the "Write" button and turning on your robot.